MATERIAL SPECIFICATION

## CHURCH OF IMMACULATE CONCEPTION

## LEAVENWORTH, KANSAS

1.	Two manual console with plain panels and roll top, glass music rack with light, and matching bench with  music compartment, pedal light; drawknobs for all stops and intra-manual couplers and tilting tablets for inter-manual couplers controlled by a self-contained Peterson solid-state combination action with 32 levels of memory and specified number of pistons, all couplers to operate through a Peterson multi-plex relay; the required 32 note pedal board built to AGO dimensions; to have the specified reversible, and the usual indicator lights and marker plates.  Console will be of the same wood as existing church furniture and finished to your finish sample.

2.	New solid-state multi-plex relay system by Peterson Electro-Musical Products to control all windchest key and stop functions.

3.	Two new fused AC/DC Astron rectifiers for providing power to the instruments electric action.

4.	New electro-pneumatic slider windchest constructed of poplar for the unenclosed Great division.  Rack boards will be of voidless plywood.

5.	New electro-pneumatic slider windchests constructed of poplar for the straight swell raniks and electro-pneumatic unit windchests for all ranks with extension and borrows.  All pouches and primary leather pneumatics in the windchests will be of tan pneumatic leather.  120 ohm chest exhaust magnets will be used on all windchests.  Muffler covers will be provided as required to make the action as noise free as possible.  Walkways for service and new building frames will be provided as required.

6.	New swell shades will be provided of voidless plywood painted black with a Peterson all electric swell engine provided to control operation movement in eight equal stages.

7.	Four new reservoirs will be provided for the various wind requirements of the instrument.  Schwimmer regulators will also be built into the slider windchests.  Concussion bellows will be provided as required on offset windchests to assure steady wind.

8.	New wind lines will be provided for all parts of the organ and new organ cable between windchests and de-multi-plex units.

9.	Zephyr quiet blower will be provided to supply the wind requirements of the organ 1 ½ HP, 220 volts single phase.

10.	New organ pipes will be provided as specified in the tonal specification.  Reeds will be constructed by A. R. Schopp’s Sons, Inc., and voiced by Fred Oyster.  Flues will be obtained from A. R. Schopp’s Sons, Inc., constructed to scales provided by the organ builder.  Spotted metal flue pipes will be of 40% tin – 60% lead.

11.	Other pipes as specified are old and in excellent condition and will be cleaned, provided with new tuning slides, zinc basses painted with our standard silver for façade pipes, and rescaled as required and revoiced for the described tonal specification.

12.	The exterior of the organ will be arranged according to a drawing approved by the Church.

13.	New case work for the instrument will be provided by Quimby Pipe Organs, Inc.